Exp No: 1

FLASHING OF LED USING AT89C51 MICROCONTROLLER USING PROTEUS

PROGRAM

ORG 0000H

UP: SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY: MOV R4,#35

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

Exp No: 2

GENERATION OF SQUARE WAVE USING PROTEUS

Program:

ORG 0000H

UP:SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY:MOV R4,#35

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

Circuit:

Exp No: 4

7 SEGMENT DISPLAY USING AT89C51 USING PROTEUS

Code:

ORG 0000H

UP:

MOV P2, #0C0H

ACALL DELAY

MOV P2, #0F9H

ACALL DELAY

MOV P2, #0A4H

ACALL DELAY

MOV P2, #0B0H

ACALL DELAY

MOV P2, #099H

ACALL DELAY

MOV P2, #092H

ACALL DELAY

MOV P2, #082H

ACALL DELAY

MOV P2, #0F8H

ACALL DELAY

MOV P2, #080H

ACALL DELAY

MOV P2, #090H

ACALL DELAY

SJMP UP

DELAY:

MOV R5, #10

H1:

MOV R4, #180

H2:

MOV R3, #255

H3:

DJNZ R3, H3

DJNZ R4, H2

DJNZ R5, H1

RET

END

Circuit:

Exp No: 9

ARDUINO BASED MANUAL ELECTRONIC COUNTER USING PROTEUS

Code:

int x0, x1, x2, x3, x4, x5, x6, x7, x8, x9;

int delay_time = 200;

void setup()

{

// Configure pin 12 as input and enable internal pull-up resistor

pinMode(12, INPUT_PULLUP);

pinMode(1, OUTPUT);

pinMode(2, OUTPUT);

pinMode(3, OUTPUT);

pinMode(4, OUTPUT);

pinMode(5, OUTPUT);

pinMode(6, OUTPUT);

pinMode(7, OUTPUT);

}

void loop()

{

// Read the pushbutton value

int sensorVal = digitalRead(12);

if (sensorVal == LOW)

{

x0 = true;

}

while (x0)

{

zero();

sensorVal = digitalRead(12);

if (sensorVal == LOW)

{

x1 = true;

x0 = false;

}

}

while (x1)

{

one();

sensorVal = digitalRead(12);

if (sensorVal == LOW)

{

x2 = true;

x1 = false;

}

}

while (x2)

{

two();

sensorVal = digitalRead(12);

if (sensorVal == LOW)

{

x3 = true;

x2 = false;

}

}

while (x3)

{

three();

sensorVal = digitalRead(12);

if (sensorVal == LOW)

{

x4 = true;

x3 = false;

}

}

while (x4)

{

four();

sensorVal = digitalRead(12);

if (sensorVal == LOW)

{

x5 = true;

x4 = false;

}

}

while (x5)

{

five();

sensorVal = digitalRead(12);

if (sensorVal == LOW)

{

x6 = true;

x5 = false;

}

}

while (x6)

{

six();

sensorVal = digitalRead(12);

if (sensorVal == LOW)

{

x7 = true;

x6 = false;

}

}

while (x7)

{

seven();

sensorVal = digitalRead(12);

if (sensorVal == LOW)

{

x8 = true;

x7 = false;

}

}

while (x8)

{

eight();

sensorVal = digitalRead(12);

if (sensorVal == LOW)

{

x9 = true;

x8 = false;

}

}

while (x9)

{

nine();

sensorVal = digitalRead(12);

if (sensorVal == LOW)

{

x0 = true;

x9 = false;

}

}

}

void zero()

{

for (int i = 1; i < 7; i++)

{

digitalWrite(i, HIGH);

}

digitalWrite(7, LOW);

delay(delay_time);

}

void one()

{

digitalWrite(1, LOW);

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, LOW);

digitalWrite(5, LOW);

digitalWrite(6, LOW);

digitalWrite(7, LOW);

delay(delay_time);

}

void two()

{

digitalWrite(1, HIGH);

digitalWrite(2, HIGH);

digitalWrite(3, LOW);

digitalWrite(4, HIGH);

digitalWrite(5, HIGH);

digitalWrite(6, LOW);

digitalWrite(7, HIGH);

delay(delay_time);

}

void three()

{

digitalWrite(1, HIGH);

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, LOW);

digitalWrite(6, LOW);

digitalWrite(7, HIGH);

delay(delay_time);

}

void four()

{

digitalWrite(1, LOW);

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, LOW);

digitalWrite(5, LOW);

digitalWrite(6, HIGH);

digitalWrite(7, HIGH);

delay(delay_time);

}

void five()

{

digitalWrite(1, HIGH);

digitalWrite(2, LOW);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, LOW);

digitalWrite(6, HIGH);

digitalWrite(7, HIGH);

delay(delay_time);

}

void six()

{

digitalWrite(1, HIGH);

digitalWrite(2, LOW);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, HIGH);

digitalWrite(6, HIGH);

digitalWrite(7, HIGH);

delay(delay_time);

}

void seven()

{

digitalWrite(1, HIGH);

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, LOW);

digitalWrite(5, LOW);

digitalWrite(6, LOW);

digitalWrite(7, LOW);

delay(delay_time);

}

void eight()

{

digitalWrite(1, HIGH);

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, HIGH);

digitalWrite(6, HIGH);

digitalWrite(7, HIGH);

delay(delay_time);

}

void nine()

{

digitalWrite(1, HIGH);

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, LOW);

digitalWrite(6, HIGH);

digitalWrite(7, HIGH);

delay(delay_time);

}

Components Required:

- Ardino

- Button

- 7 segment light

- ground

Circuit:

Exp No: 6

ARDUINO TO 16x2 LCD DISPLAY USING PROTEUS

Code:

#include <LiquidCrystal.h>

// LCD connections: RS, E, D4, D5, D6, D7

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

void setup()

{

lcd.begin(16, 2);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("HEY");

lcd.setCursor(0, 1);

lcd.print("ECE DEPARTMENT");

}

void loop()

{

// Keep the text stationary

}

Circuit:

Components Reqiured:
1.ardino

2.lcd1

3.ground

4.ardino based